# Problem Set 02 - Clock Interrupts

## 1. Choosing the duration of the quantum

Consider a time-sharing system where the duration of a quantum is denoted q (in ms) and where context switching overhead is denoted 𝜎 (in ms).

### 1.1.

Assuming that tasks always spend their entire quantum, what percentage of time gets spent switching by the system?

The system described in this question is periodical with P = q + 𝜎

The percentage of time spent switching is 𝜎 / (q + 𝜎)

NB: This formula is valid only if every task always spends its entire quantum

### 1.2.

Let q = 100 ms and 𝜎 = 1 ms.

The switching overhead gets charged to the evicted task.

What is the total running time (CPU consumption time, including the switch) of a task that requires 20 ms of computation time? Of a task that requires 500 ms of computation time?

What is the percentage of time spent by each of these in context switches?

The first task incurs a single switch upon its termination. Its total execution time is 21 ms.

The second task incurs 5 switches, for a total exec time of 505 ms.

T1 incurs 1/21 ~ 5% switching overhead, while T2 incurs 5/505 ~ 1% overhead

### 1.3.

What are the main criteria of selection for the duration of the quantum?

The duration of the switch is an important factor. Choosing a quantum duration that is close to the switch duration increases the overhead.

Increasing the duration of the quantum decreases the impact of the overhead: 𝜎 / (q + 𝜎)

However, this only works if tasks spend the whole quantum. A long quantum penalizes tasks with short computation times or frequent I/Os to the benefit of tasks with long computation times.

## 2. Managing a timer interrupt

Consider the JB007 1MHz processor that has a clock register Rtempo. Rtempo gets decremented automatically every microsecond both in user and kernel mode. When its value reaches 0, Rtempo triggers a timer interrupt. Rtempo uses a complement of two binary representation (the most significant bit represents the sign: 0 for positive, 1 for negative), and gets initialized automatically to RTMAX (all bits set to 1, except the most significant bit set to 0).

The clock interrupt is used to:

- maintain universal time in a UCR (Universal Clock Register) word,

- enforce quantum-based timesharing, and

- keep count of the computation time CT of each task. When its CT value reaches a variable threshold value CTMAX, the task is destroyed.

### 2.1.

What should be the minimum size of Rtempo if the interval between timer interrupts is 10 ms?

1 𝛍s = 10-6 s

10 ms = 10-2 s

There are 10-2/10-6 = 104 clock ticks to account for.

213 < 10000 < 214 => 14 bits required to represent 104 ticks.

Two's complement representation to account for negative values adds 1 bit

Rtempo must be at least 15 bits long.

### 2.2

Why is it essential (a) to decrement Rtempo both in kernel and user mode, and (b) to keep decrementing after it reaches 0?

Because time does not stop.

Since the clock interrupt is used to maintain universal time, the time spent in kernel mode and after reaching 0 needs to be accounted for when updating the UCR

The following handler (clock interrupt service routine) runs in kernel mode upon every timer interrupt:

Mask all interrupts

Save the context of running task T1

T1.CT += RTMAX - Rtempo

if (T1.CT ≥ CTMAX)

Terminate T1

Elect new task T2

Load the context of T2

if (T2 is a new task)

T2.CT = 0

UCR + = RTMAX - Rtempo;

Rtempo = RTMAX;

Unmask all interrupts

### 2.3.

Why is it essential to mask interrupts during this handler?

Because this has to be the highest priority interrupt handler (otherwise we lose track of time!)

If another handler can override the clock ISR, then context switching can become infinite (eg. I/O call in the overriding handler). If Rtempo overflows (reaches below its minimum negative value: 111...1 => 011...1), then the formula to update UCR sets the wrong time.

### 2.4.

Why is it crucial that the handler of the timer interrupt be kept as short as possible?

What should be the maximum processing time of the handler to maintain a consistent time management?

This follows from the previous answer. Masking all other interrupts prevents from dealing with other important events. The ISR computation must remain shorter than the capacity of Rtempo to guarantee timekeeping.

The processing time of the handler cannot exceed the capacity of Rtempo in the negatives.

### 2.5.

What are the pros and cons of always initializing register Rtempo to the same value RTMAX?

Pro

Simple to maintain: the UCR update is always relative to RTMAX

Con

Fixes the minimum frequency for events handling

eg. if RTMAX = 10 ms, then the system cannot monitor events every 1 ms

On the other hand, setting RTMAX too small increases the overhead.

eg. handling context switches upon every ISR execution is highly cost-ineffective

The general idea is to maintain a high frequency for clock interrupts while allowing larger quantums. Hence context switches should occur every N interrupts. This is how Unix works.

## 3. Quantum and tick

Unix uses the concept of tick to express the moment when the clock register value falls to 0.

Some tasks of the clock interrupt handler are performed upon each tick, while others get performed every N ticks (N depends on the task to deal with). For example, current Unix Systems enforce a quantum duration of 100 ms and a tick duration TCK of 10 ms. Operations performed upon every tick include updating the universal time and the computation time of the running task, as well as the triggering of alarms (awakening of suspended tasks, packet retransmission, ...).

### 3.1.

Modify the clock interrupt service routine to handle a switch every 100 ms with an interval between two ticks of 10 ms.

Mask all interrupts

nticks++

Save the context of running task T1

T1.CT += TCK - Rtempo

if ((T1.CT ≥ CTMAX) || (nticks == 10))

if (T1.CT ≥ CTMAX)

Terminate T1

nticks = 0

Elect task T2

Load the context of T2

if (T2 is a new task)

T2.CT = 0

else

Restore (load) the context of T1

UCR + = TCK - Rtempo;

Rtempo = TCK;

Unmask all interrupts

Every time a process gets evicted, the system adjusts its priority by applying the formula:

p_pri = p_user + p_cpu + p_nice

where p_pri is the priority of the task, p_user is the initial task priority assigned to the user, p_cpu is the computation time of the task (in number of ticks), and the setting p_nice is a value set by the user via the nice command.

### 3.2.

Modify your answer to question 3.1 so as to enforce this new policy.

Mask all interrupts

nticks++

Save the context of running task T1

T1.CT += TCK - Rtempo

if ((T1.CT ≥ CTMAX) || (nticks == 10))

if (T1.CT ≥ CTMAX)

Terminate T1

else

T1.p_pri += 10

nticks = 0

Elect task T2

Load the context of T2

if (T2 is a new task)

T2.CT = 0

else

Restore (load) the context of T1

UCR + = TCK - Rtempo;

Rtempo = TCK;

Unmask all interrupts

### 3.3

Most OSes pretend that the precision of software alarms is down to the microsecond. What is the true precision of these alarms?

The true precision of an OS is the tick, usually between 1 and 10ms.

### 3.4.

How can we measure the time spent by the task in user mode and system mode?

Maintain 2 counters p_utime and p_stime.

Upon every clock interrupt, check the execution mode and increment the corresponding counter. Statistical errors can be quite large for small tasks, but if enough ticks occur during the computation time of the task, then the estimate will be very close to the reality.